BASEMENT SLANT-CEILING STORAGE SHELF

Build Plan — v2, follows the ceiling line

# Project Overview

This is a freestanding, open-shelf storage unit built for the awkward slant-ceiling nook in the basement — the space next to the stairs where the ceiling drops from about 7 feet down to about 4 feet over a 5-foot run. Original brief: cheap, quick, open shelving for bulk household stock (paper towels, toilet paper, pantry overflow), similar to the cold-storage shelving already in use elsewhere. It needed to be freestanding rather than built-in, since the basement will eventually get finished and this unit will need to come out for that work.

This version follows a hand-sketched revision: instead of building one flat-topped box that ignores the extra height on the tall side of the slope, the design now stacks a second, shorter “topper” section on top of the box at the tall end, with its posts stepped up and cut at an angle so a flat panel closes the top flush with the real ceiling line. That angled cut is now on the table because a saw or gauge capable of angled cuts is being brought into the shop for this build.

Overall footprint: 60″ wide x 12″ deep   |   Lower box height: 46″   |   Overall height, tall end: 82″

Before you cut: your original measurements were estimates (“4ft-ish,” “7ft highest”). On site, check the real slope with a speed square or angle finder before cutting anything for the topper — the post heights and the 31° angle below are both derived directly from a 36″ rise over a 60″ run. If your actual ceiling is meaningfully different, those numbers shift with it.

# Design

Two stacked sections:

- 1. Lower box — full 60″ width, flat top, 46″ tall (2″ of clearance under a true 4ft/48″ low point). Five legs — four corners plus one center-back support so the 60″ span doesn't sag under stacked paper goods — with cleats carrying four plywood shelf levels across the entire width. Two storage bays, roughly 14–15″ of clearance each.

- 2. Upper topper — sits on top of the box, but only over the tall 42″ of the width; past that point the ceiling is already down near box height and isn't worth framing. Posts step up in height to match the slope (36″ → 20″ → 11″), their tops all cut at the same 31° angle, with a flat plywood panel resting across those angled tops — that tilt is what becomes the sloped roofline, sitting flush under the real ceiling. One partial shelf sits inside the topper on the tall end, sized to where there's still enough headroom to be useful, then stops — same as the sketch.

The angle: rise 36″ over a 60″ run → 31° off horizontal (a tool that reads angle off vertical instead will call this 59° — same cut, different reference). This is a single flat-plane angle, not a compound miter: every angled cut in this plan uses the same 31° setting, all facing the same direction.

# Cut List

## Lower Box (unchanged construction — standard flat shelf)

| Piece | Qty | Material | Dimensions | Notes |
| --- | --- | --- | --- | --- |
| Legs | 5 | 2x4 | 45.25″ long | 4 corners + 1 center-back |
| Cleats | 20 | 2x4 | 3.5″ long | 4 per leg, one per shelf height |
| Shelf decks | 4 | 3/4″ plywood | 60″ x 12″ | Rest on cleats, screwed down |

Shelf heights (top surface, up from floor): 3″, 17.5″, 32″, 46″ (top).

Cleat top-edge heights (0.75″ below shelf height for plywood thickness): 2.25″, 16.75″, 31.25″, 45.25″ (flush with leg top).

## Upper Topper (new — sits on top of the box, tall end only)

| Piece | Qty | Material | Dimensions | Notes |
| --- | --- | --- | --- | --- |
| Tall posts | 2 | 2x4 | 36″, top cut at 31° | Front & back, at the tall (0″) end of the topper |
| Mid posts | 2 | 2x4 | 20″, top cut square | Front & back, 27″ in from the tall end — supports far edge of topper shelf |
| Short posts | 2 | 2x4 | 11″, top cut at 31° | Front & back, 42″ in — where the roofline meets the box top |
| Shelf-support cleats | 4 | 2x4 | 3.5″ long | Inside face of the 2 tall posts, at 20″ up |
| Topper shelf | 1 | 3/4″ plywood | 27″ x 12″ | Square-cut both ends |
| Roof panel | 1 | 3/4″ plywood | 49″ x 12″ | Flat rectangle — no bevel needed, sits tilted across the angled post tops |

Why 49″ for the roof panel and not 42″: that's the actual sloped distance (hypotenuse) between the tall posts and short posts, not the flat horizontal run — cut it long enough to fully close the top.

Cutting the angled post tops: set your saw/miter tool to 31° and cut all 4 angled posts (2 tall + 2 short) the same direction — lay them side by side before cutting to confirm the angle faces the same way on all of them. Mid posts stay square-cut; they only carry the shelf, not the roof panel.

# Plywood Layout

Every piece above comes from a single 4x8 sheet. Rip it into four 12″-wide strips (full 96″ length) with a straightedge/track guide, then crosscut in this order to avoid waste:

- Strip 1: cut a 60″ box shelf, then from the 36″ remainder cut the 27″ topper shelf (9″ scrap left over).

- Strip 2: cut a 60″ box shelf (36″ scrap).

- Strip 3: cut a 60″ box shelf (36″ scrap).

- Strip 4: cut the 49″ roof panel (47″ scrap left over — big enough to save for a small future project).

That's all 6 plywood pieces — box and topper — from one sheet.

# Wood Recommendation

- Framing (legs, posts, cleats): construction-grade SPF 2x4 studs. Cheapest option, cuts and bevels easily with a basic saw, plenty strong for this load.

- Shelf & roof panels: 3/4″ sanded pine (BC) plywood. Smoother than CDX for a few dollars more, and since the roof panel and topper shelf edges are visible on the tall end, worth the step up. CDX still works fine structurally if you want to shave the cost.

No stain or finish needed — bare wood is standard for basement storage. Skip it unless you want the look.

# Shopping List

- 6x 2x4x8' SPF studs (4 for the box, 2 for the topper) — buy a 7th as cheap insurance against a miscut angle

- 1x sheet 3/4″ plywood, 4'x8' (sanded pine recommended, CDX as the budget option)

- Wood screws: 2.5″ for cleat-to-post and post-to-box, 1.25″–1.5″ for plywood-to-cleat/post

- Wood glue (already on hand)

- Sandpaper for easing plywood edges (already on hand)

- The angle-cutting tool mentioned for this build — a compact miter saw, or an adjustable bevel gauge/protractor jig for the circular saw. Either works as long as it holds a clean 31°, not just a fixed 45°.

# Tools Needed

- Circular saw + straightedge/track guide (rips, crosscuts, roof panel and box shelf cuts)

- Angle-cutting tool set to 31° (post tops — see note above on fixed-45° miter boxes not being enough here)

- Stop block + clamp (repeatable cleat and shelf-length cuts)

- Drill/driver + pilot-hole bit

- Tape measure, speed square (also useful for verifying the 31° angle on site), clamps

- Palm sander (ease cut edges, 120 grit, staying bare wood)

# Step-by-Step

## Box (build first)

- Verify the box side of the space clears the actual ceiling low point. Adjust the 46″ target if your measurement is off.

- Cut 5 legs to 45.25″ (2x4x8' studs, 2 per stud).

- Mark cleat heights on one leg (2.25″, 16.75″, 31.25″, 45.25″), clamp all 5 bottoms-flush, transfer marks across all of them with a speed square.

- Cut 20 cleats at 3.5″ with a stop block.

- Glue + screw cleats to the inside face of each leg at its marked heights.

- Stand the 4 corner legs at the footprint corners (60″x12″) plus the center-back leg at the 30″ mark.

- Cut the plywood per the layout above.

- Screw down the bottom shelf first, check the frame is square (diagonal measurements should match), then work up through the remaining 3 shelves.

## Topper (build on top of the box, tall end)

- Cut the 2 tall posts (36″), 2 mid posts (20″), 2 short posts (11″). Set the angle tool to 31° and cut the tops of the 2 tall + 2 short posts (mid posts stay square).

- Stand the tall posts at the very tall end of the box top (front and back, flush with the box's tall-end corners), the mid posts 27″ in from there, and the short posts 42″ in — screw all 6 down into the box top/legs below.

- Screw the 4 shelf-support cleats to the inside faces of the 2 tall posts at 20″ up.

- Set the 27″x12″ topper shelf on the tall-post cleats and the mid-post tops, screw down.

- Lay the 49″x12″ roof panel across the angled tops of the tall and short posts — it should sit flat, tilted at the slope. Screw down into all 4 post tops.

- Sand all exposed plywood edges (120 grit).

- Move the whole unit into the nook and check the roof panel sits parallel to the real ceiling — a gap of an inch or so is fine (that's the built-in clearance margin), but if it's way off, that's the cue to field-adjust an angle or a height before calling it done.

# Difficulty & Time

Difficulty: Beginner-Intermediate — the box is straightforward butt-joint work; the topper adds one repeated angled cut (31°, same setting for all 4 posts) and a bit more layout care.

Time: A full day. Roughly 1.5 hrs cutting box lumber + plywood, 1 hr cutting topper posts/angles, 2.5–3 hrs assembly (box first, then topper), plus a final fit-check once it's in place.

# Cost Estimate

| Item | Qty | Est. unit cost | Est. total |
| --- | --- | --- | --- |
| 2x4x8' SPF stud | 7 | ~$4.50 | ~$32 |
| 3/4″ plywood, 4x8 sheet (sanded pine) | 1 | ~$60 | ~$60 |
| TOTAL ESTIMATED COST |  |  | ~$92 |

Budget version with CDX plywood (~$48/sheet): ~$80 total.

This doesn't include whatever the angle-cutting tool costs if it's being bought rather than borrowed — worth pricing a basic compact miter saw before committing, since that could end up the biggest line item in the whole project. Lumber/plywood prices are Home Depot/Lowe's-range estimates for 2026 — check Farr West/Ogden pricing before buying.

# Image Prompt

Copy this straight into ChatGPT, Sora, or Midjourney to see a render of the finished piece:

Photoreal furniture/product photography of a freestanding basement storage shelving unit made of bare pine 2x4 framing and sanded pine plywood shelves, natural unfinished wood color. The unit has two stacked sections: a lower flat-top rectangular shelf box about 5 feet wide, 1 foot deep, and just under 4 feet tall with three open shelf levels, and on top of it at one end, a shorter angled-top topper section that steps up in height and follows a sloped ceiling line, with one partial shelf inside the topper and a flat sloped panel closing its top at roughly a 30 degree angle. Visible wood screws at the corner posts and cleats, minimalist utilitarian construction, exposed square-cut lumber legs. Shelves stocked with bulk paper towels, toilet paper multipacks, and pantry overflow items in cardboard boxes. The unit sits in an unfinished basement storage nook directly under a real slanted ceiling that the topper's angled top closely follows, exposed stud walls and insulation visible in the background, concrete floor, single overhead bulb lighting, slightly moody warm shop lighting, wide angle shot