# Mobile Shop Cart

Built from workbench leftovers — a 3-tier rolling cart that tucks under your new workbench when not in use.

# Materials On Hand

- 1 OSB panel, roughly 48" x 48" — leftover from the workbench build

- 1 long 2x4, roughly 8 ft (96")

- 1 shorter 2x4 cutoff, roughly 36-42"

- ~9 small 2x4/2x6 scrap blocks, 3-6" each

Note: dimensions above are estimated from photos. Measure your actual boards and the OSB panel before cutting — the cut list below assumes a 96" board and a ~48"x48" panel. If yours run shorter, trim the cleat/leg counts accordingly.

# The Build

A 24" x 24" footprint, 3-tier rolling cart, roughly 27" tall to the top shelf once casters are on. Corner blocks from your scrap pile handle the racking strength so you don’t need a full frame — matches the exposed-fastener look of the workbench you just built.

## Cut List

| Piece | Qty | Dimension | Source |
| --- | --- | --- | --- |
| Legs | 4 | 24" long (2x4) | Long 2x4 (~96") — cuts exactly, zero waste |
| Shelf cleats | 8 | 3" long (2x4) | Short 2x4 cutoff |
| Corner brace blocks | 8 | ~3-4" (as-is) | Scrap block pile — 1 block left over |
| Shelves (top/mid/bottom) | 3 | 24" x 24" | OSB sheet, cut into 4 equal quadrants |
| Spare shelf panel | 1 | 24" x 24" | OSB sheet — 4th quadrant, bonus for a future wall shelf |

## Shopping List & Cost

| Item | Qty | Est. unit cost | Est. total |
| --- | --- | --- | --- |
| Lumber & OSB (already on hand) | — | $0 | $0 |
| 2" swivel casters (2 locking, 2 fixed) | 4 | $7-9 | $28-36 |
| Wood screws (2.5" + 1.25") | 1 box each | $4-5 | $8-10 |
| Wood glue | 1 | $5 | $5 |
| Finish (optional, to match workbench) | 1 can | $12-15 | $12-15 |
| Total estimated cost |  |  | $28-51 buying everything; ~$28-36 if you already have screws/glue |

Prices are Home Depot/Lowe’s-range estimates — worth a quick check against Farr West/Ogden pricing before buying. You likely already have screws and glue left from the workbench build, which puts the real number closer to $30.

## Tools Needed

- Circular saw + speed square (as a cutting guide)

- Drill/driver

- Tape measure and pencil

- Clamps

- Sander or sandpaper (120-150 grit)

- Wood glue

# Step-by-Step

- Confirm actual lengths of your two 2x4 pieces and the OSB panel before cutting anything — this plan assumes a 96" board, a 36-42" cutoff, and a 48"x48" panel.

- Cut 4 legs at 24" from the long 2x4, using the speed square as a saw guide for square cuts.

- Cut 8 cleats at 3" from the shorter 2x4 cutoff.

- Cut the OSB panel into 4 equal 24" x 24" quadrants: mark a center line both directions with a pencil and speed square, then cut with the circular saw.

- Mark cleat positions on each leg: one cleat face 3" up from the foot (bottom shelf), one 12" up (middle shelf). Drill 1/8" pilot holes and attach cleats with 2.5" screws, two per cleat.

- Stand the 4 legs up in a 24" x 24" square. Set the bottom OSB panel on the bottom cleats, check both diagonals measure the same (confirms square), then screw down through the panel into the cleats and leg tops with 1.25" screws, 2-3 per side.

- Repeat for the middle shelf on the middle cleats.

- Set the top shelf flush on top of the 4 legs and screw down into each leg, 2 screws per corner.

- Glue and screw a corner brace block into each of the 4 top inside corners, where leg meets the underside of the top shelf. This is what stops the cart from racking side to side — don’t skip it.

- Flip the cart upside down. Center a caster on the bottom face of each leg, pilot-drill, and screw down. Add washers if the caster screw holes are oversized for the 2x4 face.

- Flip right-side up. Sand all edges and corners smooth — OSB edges are rough and splintery straight off the saw.

- Optional: finish with wipe-on poly or leftover stain from the workbench to match, 1-2 coats.

## Difficulty & Time

Easy-moderate. Plan on 3-4 hours including cutting, assembly, and finish drying — a single afternoon build.

# Image Prompt

Copy-paste this into ChatGPT, Sora, or Midjourney to see a render of the finished cart:

A small mobile shop cart, 24 inches square and about 27 inches tall, built from unfinished pine 2x4 legs and OSB (oriented strand board) shelves on three tiers, with visible wood screws at each joint and four black swivel casters at the base. Natural, unfinished wood tones. Photographed in a home garage workshop next to a matching handmade wood workbench with a drill, hammer, and tape measure resting on top, natural daylight through an open garage door, photoreal furniture/product photography style, shallow depth of field.