# White Plywood & 2x4 Side Chair

Build Plan — The Insurance Center Workshop Project

# The Verdict

Yes, this works — with two adjustments. A 1/2" plywood seat on its own will flex noticeably under body weight over a 16" span, so the plan adds a center support cleat underneath (cheap, uses scrap, five extra minutes). And your one full 2x4 covers the four legs and not much else — the rails, backrest support, and corner blocks need to come from scrap or a second 2x4. Unless your scrap pile has close to 7 more linear feet of 2x4 in it, budget for one more 8-foot stud. It's a $5 problem, not a redesign problem.

The two-tone look (stained/oiled frame, natural plywood seat and back) is a good call — it reads modern and hides the fact that this is a screw-and-glue build instead of joinery.

# Design Specs

- Overall height: 32"

- Seat height: 17"

- Seat size: 16" wide x 16" deep

- Backrest: 15" tall above the seat, vertical (no recline — keeps every cut a 90° crosscut, no angled joinery needed for a circular saw)

- Assumes your plywood offcut is at least 16" x 32" (about 3.5 sq ft). If it's smaller, scale the seat down first — tell me the actual piece size and I'll re-cut the numbers.

# Cut List

| Part | Qty | Length | Material | Source |
| --- | --- | --- | --- | --- |
| Back legs / backrest posts | 2 | 32" | 2x4 (1.5"x3.5") | Full 2x4 |
| Front legs | 2 | 17" | 2x4 (1.5"x3.5") | Full 2x4 |
| Front seat rail | 1 | 13" | 2x4, ripped or on-edge | Scrap |
| Back seat rail | 1 | 13" | 2x4, ripped or on-edge | Scrap |
| Side seat rails | 2 | 13" | 2x4, ripped or on-edge | Scrap |
| Upper back rail | 1 | 13" | 2x4 | Scrap |
| Corner glue blocks | 4 | 3" | 2x4 offcut | Scrap |
| Center seat support cleat | 1 | 13" | 2x4 offcut, ripped narrow | Scrap |
| Seat panel | 1 | 16"x16" | 1/2" plywood | On hand |
| Backrest panel | 1 | 16"x15" | 1/2" plywood | On hand |

2x4 running total: legs 98" + rails 65" + corner blocks 12" + center cleat 13" ≈ 188" ≈ 15.7 linear feet. Your full 2x4 gives you 8 feet. Measure your scrap pile — if it doesn't add up to roughly 7–8 more feet, grab one more 8-foot 2x4.

# Shopping List (only what you don't already have)

- 1x 2x4x8 pine stud (covers the shortfall on rails/blocks/cleat)

- 2.5"–3" exterior wood screws (rail-to-leg joints)

- 1.25" wood screws (seat/back panel to frame)

- Wood glue (every joint gets glue, not just screws)

- Stain or wood oil for the frame (your call — oil is more forgiving for a first-timer)

- Clear coat or wipe-on poly for the plywood (protects the light color, keeps it from yellowing/graying with use)

- Optional: iron-on edge banding for the plywood edges — 1/2" ply shows raw layers on the seat front edge, which reads cheap if left bare

# Tools Needed

- Circular saw + straightedge/track guide

- Drill/driver with pilot-hole and countersink bits

- Wood glue, clamps

- Tape measure, speed square

- Clamped stop block — use this for every repeated-length cut (the 4 rails, the corner blocks) so all four come out identical without measuring each one individually

- Sandpaper (120 then 220 grit)

# Build Steps

- Cut the 4 legs to length (2 at 32", 2 at 17") from the full 2x4. Cut the 4 seat rails, the upper back rail, the 4 corner blocks, and the center cleat from your scrap — or the second 2x4 if scrap comes up short. Set up a stop block for the rails and corner blocks so all four rails match exactly.

- Dry-fit the seat frame: front rail, back rail, and 2 side rails between the 4 legs at 17" from the floor (bottom edge of rail flush with 17" mark). The rails sit against the inside faces of the legs.

- Glue and screw each rail to each leg: 2 screws per joint, driven through the leg into the rail end, pilot-drilled first so the 2x4 doesn't split. Check the frame is square (measure both diagonals — they should match) before the glue sets.

- Glue and screw a corner block into each of the 4 inside corners of the seat frame, screwed into both the leg and the rail. This is what actually keeps the frame from racking under weight — don't skip it.

- Attach the upper back rail across the 2 back legs, 15" above the seat rail. This gives the backrest panel something to screw into at the top.

- Let all glue joints cure at least 30–45 minutes (longer if your shop is cold) before moving the frame around.

- Cut the seat panel (16"x16") and backrest panel (16"x15") from your plywood.

- Cut the center support cleat to fit snugly between the front and back rails, running front-to-back under the seat panel's midpoint. Glue and screw it to the underside of the seat panel first, then screw that assembly down into the seat rails.

- Screw the seat panel down into the 4 rails from above (countersunk screws, or screw up from underneath through the rails if you want zero visible fasteners on top).

- Screw the backrest panel to the 2 back legs and the upper back rail.

- Sand every edge and corner — 120 grit first, then 220. Round over the plywood edges enough that they don't feel sharp.

- Finish: stain or oil the 2x4 frame (2 coats, drying time per product — usually 4–6 hours between coats). Clear-coat the plywood seat and back separately, keeping finish off any surfaces that are still masked or clamped.

# Difficulty & Time

Intermediate — nothing here requires a tool you don't have, but square, accurate crosscuts with a circular saw + stop block take patience. Realistic timeline: 2–3 hours cutting and dry-fitting, 1–2 hours assembly (spread across glue-cure waits), and a separate day for finishing since stain/oil and poly both need dry time between coats and before use. Plan on a weekend, not an evening.

# Cost Estimate

| Item | Qty | Est. unit cost | Est. total |
| --- | --- | --- | --- |
| Lumber already on hand (1 full 2x4 + scraps, 1/2" ply) | — | $0 | $0 |
| 2x4x8 pine stud (buy 1 more) | 1 | $4–$6 | $4–$6 |
| Iron-on edge banding (optional, cleans up ply edges) | 1 roll | $6–$8 | $6–$8 |
| Estimated total (lumber only — see note) |  |  | $10–$14 |

Cost estimate covers lumber only — screws, glue, stain/oil, and clear coat aren't priced since those are shop staples you keep stocked. Prices are national home-center ballpark — worth a quick check against your Farr West/Ogden store before buying.

Shopping list below still lists the consumables you'll need for this build (screws, glue, finish) even though they're not priced out — grab them from your stock or restock as needed.

# Image Prompt — Copy/Paste Into Your Image Tool

Photorealistic product photography of a handmade modern side chair, 32 inches tall, 16 inch square seat. The seat and backrest are light natural-colored 1/2 inch plywood with visible edge grain, finished with clear satin coat. The frame -- four legs and seat rails -- is made of 2x4 pine, finished with a warm brown wood stain (or natural danish oil sheen), with visible exposed wood screws at the joints for a modern industrial-utilitarian look. The chair has a simple boxy silhouette: straight vertical legs, a flat square seat, and a low straight-backed backrest, no curves. Shot in a bright minimalist workshop or modern living room setting, soft natural window light, shallow depth of field, angled three-quarter view showing both the seat and the side profile of the frame.