Large-Breed Dog House Build Plan

Sized for a German Shepherd / Rottweiler mix (75–100 lb) • Shed-roof, raised-floor design • Full materials list to buy from scratch

# 1. Design Overview

Exterior footprint: 42" wide × 36" deep. Front wall 30" tall, back wall 24" tall — a 6" shed-style slope sheds rain/snow toward the back, away from the door. Roof overhangs 2" on each side and 3" front and back for weather protection. Floor is raised 3.5" off the ground on treated runners for airflow and insulation from cold/wet ground. Door is offset toward the right side of the front wall so the left portion of the wall acts as a wind block for the interior — a simple, well-tested design for outdoor breeds that handle cold reasonably well but still benefit from a draft-free sleeping area.

| Spec | Dimension |
| --- | --- |
| Exterior floor size | 42"W × 36"D |
| Front wall height | 30" |
| Back wall height | 24" (6" slope for runoff) |
| Roof overhang | 2" sides / 3" front & back |
| Floor height off ground | 3.5" (on 2×4 runners) |
| Door opening | 16"W × 22"H, arched top, offset 4" from left edge |
| Interior clear floor space | ≈ 38" × 32" — enough for a 75–100 lb dog to turn around and stretch out |

# 2. Materials List (buying from scratch)

## Plywood

- 1 sheet — 3/4" pressure-treated (or exterior-grade) plywood, 4′×8′ — floor

- 4 sheets — 1/2" exterior-grade (BC or T1-11) plywood, 4′×8′ — front, back, 2 side walls, roof (4th sheet covers waste/nesting margin — see cut list note)

## Framing lumber

- 3 — 2×4×8′ pressure-treated lumber — base runners + floor joists

- 4 — 2×2×8′ lumber — corner posts, roof cleats, door trim

## Roofing

- 1 bundle 3-tab asphalt shingles (covers up to 33 sq ft — roof is ≈ 14 sq ft) OR a 4′×6′ EPDM rubber roofing membrane for a more waterproof, low-maintenance option

- 1 box galvanized roofing nails

- 2 pieces drip-edge trim, 46" each (optional but recommended)

## Hardware & finishing

- 1 box exterior wood screws, 1⅝" (plywood-to-frame)

- 1 box exterior deck screws, 3" (frame/runner assembly)

- 1 bottle exterior wood glue (e.g., Titebond III)

- 2 heavy-duty gate hinges (optional — makes the roof removable for cleaning)

- 1 tube exterior-grade caulk

- Exterior primer + paint, or exterior stain + spar urethane sealant (2 coats)

# 3. Cut List

| Part | Qty | Dimensions | Material |
| --- | --- | --- | --- |
| Floor | 1 | 42" × 36" | 3/4" PT plywood |
| Front wall | 1 | 42" × 30" (door cut before install) | 1/2" ext plywood |
| Back wall | 1 | 42" × 24" | 1/2" ext plywood |
| Side walls | 2 | 36" × 30" tapering to 24" (cut both from one 36×30 blank) | 1/2" ext plywood |
| Roof panel | 1 | 46" × 43" (measured along the slope) | 1/2" ext plywood |
| Base runners (skids) | 2 | 36" long | 2×4 PT |
| Floor joists | 3 | 39" long | 2×4 PT |
| Corner posts, front | 2 | 30" long | 2×2 |
| Corner posts, back | 2 | 24" long | 2×2 |
| Roof cleats/ledger | 4 | cut to fit wall tops (≈36–42") | 2×2 |
| Door trim | 1 set | cut to fit 16×22" arched opening | 2×2 |

Nesting note: the front (42×30) and back (42×24) walls stack on one 1/2" sheet with room left over. The two side-wall tapers nest on a second sheet if you flip one blank end-for-end. The roof panel (46×43) needs most of a third sheet on its own because of its width. The 4th sheet is a buffer for mistakes, the door cutout offcut, or a hinged roof-access panel — you likely won't use all of it.

# 4. Tools Needed

- Circular saw or table saw (straight cuts on plywood/lumber)

- Jigsaw (door arch, roof-slope taper cuts)

- Drill/driver + countersink bit

- Tape measure, speed square, level

- Clamps (at least 4)

- Orbital sander or sanding block

- Paintbrush or roller, caulk gun

- Safety glasses + hearing protection

# 5. Build Steps

- Cut all plywood and lumber to the cut list above. Cut the door opening and the side-wall tapers now, while panels are still flat on sawhorses — much easier than cutting after assembly. Sand all cut edges.

- Build the base frame: screw the 2 runners and 3 joists together into a rectangle using 3" deck screws (pre-drill to avoid splitting). This is the skid frame the whole house sits on.

- Attach the 3/4" floor panel to the top of the base frame with 1⅝" screws every 8", pre-drilled and countersunk.

- Stand up the 2 front corner posts (30") and 2 back corner posts (24") at the floor corners. Screw and glue into the floor edge; check plumb with a level before the glue sets.

- Attach the back wall (42×24) to the back two posts and the floor's back edge. Screw every 6–8" into posts and floor.

- Attach the front wall (with door already cut) to the front two posts the same way.

- Attach both tapered side walls, screwing into the front wall edge, back wall edge, corner posts, and floor edge — this ties the whole box together and squares it up.

- Screw the 2×2 roof cleats along the top inside edge of the front, back, and side walls so the roof panel has full bearing support.

- Set the roof panel on top with a 2" overhang on each side and 3" overhang front and back. Screw down into the cleats every 8". (If using hinges for a removable roof, mount them on the back edge now instead of screwing solid.)

- Install roofing: start at the bottom edge and work up, nailing every 6", overlapping courses per the shingle manufacturer's instructions (or roll out and adhere the EPDM membrane per its instructions). Add drip edge along the front and side overhangs if using.

- Caulk all seams where the roof meets the walls, and any plywood butt joints, to block wind and water.

- Sand all edges and surfaces smooth, especially the door opening where the dog will brush against it daily.

- Prime and paint, or stain and seal, all exterior surfaces — 2 coats, letting each dry fully. This is what actually protects the plywood long-term, don't skip a coat.

- Let the finish cure 24–48 hours before adding bedding and letting the dog use it. Add straw or cedar shavings for insulation in colder months.

# 6. Notes for a German Shepherd / Rottweiler Mix

- At 75–100 lb, this dog needs more floor space than off-the-shelf "large" doghouses often provide — the 38×32" interior in this plan gives real room to turn around and lie fully stretched out, which matters more for joint comfort than people expect.

- The offset door plus solid interior wall segment blocks wind better than a centered door — worth keeping even though it's tempting to center it for looks.

- If this dog will be outside through a Utah winter, consider adding 1" rigid foam insulation between an inner and outer wall layer — happy to redo the cut list as a double-wall insulated version if you want that instead.